Single-Piece DfAM Redesign of a Liquid Rocket Injector/Nozzle Assembly

A Design-for-Additive-Manufacturing case study: propulsion calculations, baseline CAD, single-piece redesign, structural validation, and instrumentation design

# 1. Motivation

Several modern small-launch-vehicle companies have moved toward 3D-printing entire rocket engines as a single piece, eliminating the joints, fasteners, and assembly steps inherent to traditionally machined multi-part engines. This project applies that same design philosophy at a smaller, student-accessible scale: a conventionally-designed multi-part injector/nozzle assembly is redesigned as one consolidated additively-manufactured part, and the redesign is validated using hand calculations and simulation.

The project uses representative values for a generic small LOX/Kerosene test-case engine throughout — not any specific company's proprietary specifications.

# 2. Design Targets (Propulsion Calculations)

A Python script (SciPy) was used to derive design targets from a set of assumed operating parameters, representative of a small liquid engine test case.

## 2.1 Assumed Inputs

| Parameter | Value |
| --- | --- |
| Target thrust | 3,000 N |
| Chamber pressure | 20 bar (2 MPa) |
| Propellants | LOX / Kerosene (RP-1) |
| Assumed sea-level Isp | 290 s |
| Oxidizer:fuel ratio | 2.5 |
| Characteristic velocity (c*) | 1,750 m/s |

## 2.2 Calculated Design Targets

| Output | Value |
| --- | --- |
| Total propellant mass flow | 1.055 kg/s (0.753 ox + 0.301 fuel) |
| Injector pressure drop | 3.5 bar (17.5% of chamber pressure) |
| Oxidizer orifices | 49 × 1.0 mm |
| Fuel orifices | 24 × 1.0 mm |
| Throat diameter | 34.28 mm |
| Exit diameter | 64.97 mm |
| Exit Mach number | 2.54 |
| Area ratio (Ae/At) | 3.59 |
| Estimated heat flux (throat) | 19.2 MW/m² |

## 2.3 Chamber Geometry Check

A characteristic-length (L*) and contraction-ratio check was run against the as-modeled baseline chamber dimensions (Ø55mm bore, 90mm straight length). This returned L* ≈ 263mm, against a typical LOX/Kerosene design range of 800–1500mm. This indicates the chamber, as modeled, is shorter than standard practice would recommend for complete combustion residence time. This is documented as a known limitation; it does not affect the validity of the single-piece consolidation comparison, which is geometry-independent of chamber length.

# 3. Baseline Design (Multi-Part)

The baseline assembly was modeled in Autodesk Fusion 360 (Personal Use license) as three separate components, representative of a traditionally-machined engine assembly:

- Injector plate — Ø70mm disk, 10mm thick, with 73 injector holes (49 oxidizer + 24 fuel) across 5 concentric rings (Ø12/18/24/30/36mm), plus an Ø82mm mounting flange with 8 ×4.5mm clearance holes on a Ø76mm bolt circle

- Combustion chamber — Ø55mm ID, Ø65mm OD, 90mm straight length, 5mm wall

- Nozzle — converging-diverging profile, Ø55mm inlet, Ø34.28mm throat, Ø64.97mm exit, 70mm axial length

The three components were assembled using two rigid joints (injector-to-chamber, chamber-to-nozzle). Only the injector-to-chamber interface includes a bolted flange (8 × M4 clearance holes); the chamber-to-nozzle interface is a positional joint only, without modeled fasteners — noted here as an acknowledged simplification of the baseline.

# 4. Single-Piece DfAM Redesign

The redesign consolidates all three baseline components into a single revolved solid body, modeled directly from the same throat/exit/injector dimensions as the baseline, ensuring a fair comparison.

## 4.1 Design Features

- One continuous revolved body from injector face through chamber, throat, and nozzle exit — no seams, flanges, or fasteners

- Same 73-hole injector pattern (49 oxidizer + 24 fuel), cut into a solid 10mm end cap

- 6 integrated cooling channels (1.5mm liner + 3mm channel height + 2.5mm outer skin), left open at both ends to allow powder removal in a metal powder-bed process and to double as coolant ports

- 1mm fillets at the two internal direction-change corners (chamber-to-throat, throat-to-nozzle) to reduce stress concentration

## 4.2 Part-Count Comparison

| Metric | Baseline (multi-part) | Redesign (single-piece) |
| --- | --- | --- |
| Part count | 3 | 1 |
| Fastener interfaces | 2 (1 bolted, 1 joint-only) | 0 |
| Mounting holes | 8 | 0 |
| Cooling provisions | None | 6 integrated channels |

Note: the cooling channel count (6) was reduced from a production-representative value (e.g. 24) to keep the modeling scope achievable within the project timeline; this is noted as a deliberate simplification.

# 5. Structural & Thermal Validation (SimScale)

## 5.1 Structural Simulation

A static structural simulation was run on the single-piece redesign under a 20 bar (2 MPa) internal pressure load, with the material assigned as Ti-6Al-4V (yield strength ≈ 880 MPa).

| Result | Value |
| --- | --- |
| Maximum von Mises stress | 141 MPa |
| Material yield strength (Ti-6Al-4V) | 880 MPa |
| Safety factor | 6.24 |

A safety factor of 6.24 indicates the 7mm wall thickness is structurally conservative for the applied pressure load — comfortably exceeding typical aerospace practice (1.5–2.0×). This suggests a future iteration could explore wall-thickness reduction to trade some margin for mass savings.

## 5.2 Thermal Simulation — Documented Limitation

A steady-state thermal simulation was attempted to validate the Stage 1 hand-calculated heat flux estimate (19.2 MW/m², target wall temperature 800K). The initial setup (Conjugate Heat Transfer analysis) produced non-physical results (solver divergence, reporting temperatures from −1832K to 7.99×10⁵K), most likely due to an incomplete fluid-flow boundary definition inherent to the CHT analysis type — a simpler steady-state conduction analysis would have been the more appropriate choice for the boundary conditions defined.

Due to free-tier compute limits on the simulation platform, a corrected re-run was not completed within the project timeline. The Stage 1 first-order estimate is retained as the primary thermal reference for this project. This is reported transparently as a limitation rather than omitted, consistent with the project's overall approach of validating claims and documenting gaps honestly.

# 6. Sensor & Telemetry Board (KiCad)

A sensor/telemetry PCB was designed in KiCad to represent how the redesigned part could be instrumented during a hypothetical static test — directly applying an electronics/embedded-systems background to a propulsion hardware context.

## 6.1 Board Features

- ESP32 microcontroller module for sensor readout and data logging

- Chamber pressure sensor input (analog, with RC low-pass filtering)

- Two thermocouple amplifier inputs (SPI) for throat and injector temperature monitoring

- Onboard voltage regulation (barrel/terminal block input → 3.3V logic rail)

- Reset button, status LEDs, 4-corner mounting holes

The board passed design rule checks (DRC) with 0 errors. Schematic, PCB layout, Gerber/drill files, bill of materials, and a 3D render are included in the project repository.

# 7. Known Limitations & Future Work

- Chamber L* (263mm vs. a typical 800–1500mm range) — the as-modeled chamber is shorter than standard LOX/Kerosene practice would recommend; does not affect the validity of the part-count/consolidation comparison

- Thermal simulation incomplete — documented above; a corrected steady-state conduction run is the logical next step

- Cooling channel count reduced to 6 from a production-representative value for modeling-scope reasons

- Coolant inlet/outlet port routing was not modeled

- Print-readiness analysis (slicer-based orientation and support-structure check) was scoped as future work. The part would realistically be manufactured via metal powder-bed fusion rather than FDM; a slicer tool approximates general printability concepts (overhangs, orientation) but does not simulate the actual metal additive manufacturing process

# 8. Conclusion

This project demonstrates the practical reasoning behind single-piece additive manufacturing for rocket engine hardware: a 3-part, 8-fastener baseline assembly was redesigned as a single consolidated body with zero fasteners and integrated cooling — while remaining dimensionally consistent with hand-calculated propulsion design targets and passing a structural validation check with significant margin (SF = 6.24). Equally important, every limitation encountered along the way — the short chamber L*, the diverged thermal simulation, the reduced channel count — was investigated, understood, and documented rather than hidden, reflecting an engineering approach grounded in validating claims rather than assuming them.

Tools used: Python (SciPy) for propulsion calculations; Autodesk Fusion 360 for CAD modeling and built-in simulation; SimScale for structural and thermal validation; and KiCad for sensor/telemetry PCB design.